Public Quarterly Report

Date of Report: 4th Quarterly Report, October 1, 2023

Contract Number:  693JK32210009POTA

Prepared for: Government Agency: DOT and Co-funders

Project Title: Innovative Leak Detection Methods for Gas and Liquid Pipelines

Prepared by: Pipeline Research Council International, Inc.

Contact Information: Gary Choquette: gchoquette@prci.org phone: 402-917-3395

For quarterly period ending: September 30, 2023

1: Items Completed During this Quarterly Period:

| Item # | Task # | Activity/Deliverable | Title | Federal Cost | Cost Share |
| --- | --- | --- | --- | --- | --- |
| 6 | 2 | Inline flow measurement selection guidelines | Submitted with last quarterly report | $80,704 | $40,352 |
| 9 | 3 | Host site review checklist development | Summary of the review and issues to be included in quarterly report | $9,984 | $4,992 |
| 10 | 2 | Validation of overlapping zone balancing and statistical flow calibration on simulated systems | Summary of the review in quarterly report | $24,696 | $12,348 |
| 11 | 1 | Quarterly project management & status update | Submit 3nd quarterly report | $4,866 | $2,433 |

2: Items Not Completed During this Quarterly Period:

| Item # | Task # | Activity/Deliverable | Title | Federal Cost | Cost Share |
| --- | --- | --- | --- | --- | --- |
| 7 | 4 | PODS draft SCADA interface data model | Results to be included in quarterly report | $23,796 | $23,796 |

3: Project Financial Tracking During this Quarterly Period:

Note that this chart reflects Federal share only.

Invoicing for completed tasks to date are expected in Q4 2023.

4:  Project Technical Status

## Field Audit

The field audit of the host site was performed. The audit was largely clean but a few minor issues were identified specifically:

- Some pipeline pack calculations did not appear to be correct. Specifically, the pack differential flow and the corresponding energy did not always have the same sign convention, (i.e., the difference in volume would show a value of 0.696 but the corresponding energy would be -0.867). This is not physically possible and shows a potential error in the methods used to calculate line pack differences.

- Some very small volume meters have very high or very low apparent energy content. This appears to be related to using integer values for these meters. The overall impact is low due to the very small volumes involved.

Further follow-up with the field on the identified issues is required.

## Flow Pattern Matching

Flow pattern matching analysis was completed on a simulated system. The concept is to look for changes in the differences between the sum of the receipt volumes in a zone and the sum of the related deliveries as the individual meters within the zone change volumes. A regression or a solver is then used to estimate the amount of error required at each meter (excluding boundary measurement meters) that would minimize the sum of the square of the zone flow receipt/delivery differences. As an example, with a zone that has five receipt meters (Meters A through E) and one boundary meter (F), take the following 12 different flow cases:

| Case | Meter | Actual | Apparent | Actual Error |
| --- | --- | --- | --- | --- |
| 1 | A | 5.000 | 5.179 | 3.6% |
| 1 | B | 5.000 | 4.871 | -2.6% |
| 1 | C | 5.000 | 5.300 | 6.0% |
| 1 | D | 5.000 | 4.771 | -4.6% |
| 1 | E | 5.000 | 6.513 | 30.3% |
| 1 | F | 25.000 | 24.745 | -1.0% |
| 2 | A | 2.500 | 2.569 | 2.8% |
| 2 | B | 5.000 | 4.932 | -1.4% |
| 2 | C | 5.000 | 5.261 | 5.2% |
| 2 | D | 5.000 | 4.781 | -4.4% |
| 2 | E | 5.000 | 6.534 | 30.7% |
| 2 | F | 22.500 | 22.615 | 0.5% |
| 3 | A | 0.000 | 0.000 | 0.0% |
| 3 | B | 5.000 | 4.892 | -2.2% |
| 3 | C | 5.000 | 5.295 | 5.9% |
| 3 | D | 5.000 | 4.787 | -4.3% |
| 3 | E | 5.000 | 6.451 | 29.0% |
| 3 | F | 20.000 | 19.779 | -1.1% |
| 4 | A | 5.000 | 5.093 | 1.9% |
| 4 | B | 5.000 | 4.909 | -1.8% |
| 4 | C | 5.000 | 5.219 | 4.4% |
| 4 | D | 5.000 | 4.764 | -4.7% |
| 4 | E | 5.000 | 6.448 | 29.0% |
| 4 | F | 25.000 | 24.702 | -1.2% |
| 5 | A | 5.000 | 5.075 | 1.5% |
| 5 | B | 5.000 | 4.832 | -3.4% |
| 5 | C | 5.000 | 5.265 | 5.3% |
| 5 | D | 5.000 | 4.743 | -5.1% |
| 5 | E | 2.500 | 3.283 | 31.3% |
| 5 | F | 22.500 | 22.361 | -0.6% |
| 6 | A | 5.000 | 5.157 | 3.1% |
| 6 | B | 5.000 | 4.910 | -1.8% |
| 6 | C | 5.000 | 5.189 | 3.8% |
| 6 | D | 5.000 | 4.706 | -5.9% |
| 6 | E | 1.000 | 1.287 | 28.7% |
| 6 | F | 21.000 | 21.213 | 1.0% |
| 7 | A | 5.000 | 5.069 | 1.4% |
| 7 | B | 5.000 | 4.924 | -1.5% |
| 7 | C | 5.000 | 5.313 | 6.3% |
| 7 | D | 5.000 | 4.752 | -5.0% |
| 7 | E | 0.000 | 0.000 | 0.0% |
| 7 | F | 20.000 | 20.124 | 0.6% |
| 8 | A | 6.000 | 6.126 | 2.1% |
| 8 | B | 4.000 | 3.943 | -1.4% |
| 8 | C | 7.000 | 7.366 | 5.2% |
| 8 | D | 3.000 | 2.854 | -4.9% |
| 8 | E | 5.000 | 6.436 | 28.7% |
| 8 | F | 25.000 | 25.218 | 0.9% |
| 9 | A | 10.000 | 10.132 | 1.3% |
| 9 | B | 5.000 | 4.876 | -2.5% |
| 9 | C | 5.000 | 5.219 | 4.4% |
| 9 | D | 5.000 | 4.792 | -4.2% |
| 9 | E | 5.000 | 6.465 | 29.3% |
| 9 | F | 30.000 | 29.722 | -0.9% |
| 10 | A | 5.000 | 5.153 | 3.1% |
| 10 | B | 10.000 | 9.697 | -3.0% |
| 10 | C | 5.000 | 5.304 | 6.1% |
| 10 | D | 5.000 | 4.697 | -6.1% |
| 10 | E | 5.000 | 6.578 | 31.6% |
| 10 | F | 30.000 | 30.310 | 1.0% |
| 11 | A | 5.000 | 5.188 | 3.8% |
| 11 | B | 5.000 | 4.818 | -3.6% |
| 11 | C | 10.000 | 10.433 | 4.3% |
| 11 | D | 5.000 | 4.768 | -4.6% |
| 11 | E | 5.000 | 6.536 | 30.7% |
| 11 | F | 30.000 | 29.784 | -0.7% |
| 12 | A | 5.000 | 5.067 | 1.3% |
| 12 | B | 5.000 | 4.830 | -3.4% |
| 12 | C | 5.000 | 5.279 | 5.6% |
| 12 | D | 10.000 | 9.404 | -6.0% |
| 12 | E | 5.000 | 6.437 | 28.7% |
| 12 | F | 30.000 | 30.318 | 1.1% |

Note that:

- The flows for each case are relatively the same except one meter (and that meter isn’t the same in each case) has a change in flow.

- All Apparent flows have an error based on a slope plus bias plus noise (random noise nominally in the range of +/- 1%). I.e., the error is not a constant for each meter.

- One meter (E) has the largest error of all the meters. Nominally, it is reading 30% higher than it should.

The corresponding lost and unaccounted (LAUF) for each flow case are:

| Case | R | D | LAUF |
| --- | --- | --- | --- |
| 1 | 26.634 | 24.745 | 7.09% |
| 2 | 24.077 | 22.615 | 6.07% |
| 3 | 21.426 | 19.779 | 7.69% |
| 4 | 26.432 | 24.702 | 6.55% |
| 5 | 23.197 | 22.361 | 3.60% |
| 6 | 21.248 | 21.213 | 0.17% |
| 7 | 20.058 | 20.124 | -0.33% |
| 8 | 26.726 | 25.218 | 5.64% |
| 9 | 31.485 | 29.722 | 5.60% |
| 10 | 31.430 | 30.310 | 3.56% |
| 11 | 31.744 | 29.784 | 6.17% |
| 12 | 31.017 | 30.318 | 2.25% |

The case with the smallest LAUF is Case 6 followed by case 7 which is the case where there is very low flow and no low at meter E for the respective cases. This is an indication that meter E has an outsized effect on the overall system flow balance.

To test if a method could be generated to systematically identify which meters are contributing to the LAUF and their approximate magnitude of impact, a solver routine was used to estimate the error adjustments for all the meters (excluding boundary meters) needed to minimize the sum of the square of the difference between receipts and deliveries for the zone. The adjusted LAUF are:

| Case | RErrAdj | DErrAdj | LAUFAjd |
| --- | --- | --- | --- |
| 1 | 25.094 | 24.745 | 1.39% |
| 2 | 22.587 | 22.615 | -0.12% |
| 3 | 20.004 | 19.779 | 1.13% |
| 4 | 24.918 | 24.702 | 0.87% |
| 5 | 22.525 | 22.361 | 0.73% |
| 6 | 21.116 | 21.213 | -0.46% |
| 7 | 20.270 | 20.124 | 0.72% |
| 8 | 24.791 | 25.218 | -1.72% |
| 9 | 29.871 | 29.722 | 0.50% |
| 10 | 30.073 | 30.310 | -0.79% |
| 11 | 29.806 | 29.784 | 0.07% |
| 12 | 29.984 | 30.318 | -1.11% |

The estimated error calculated from the solver methods is now compared to the actual average error:

| Meter | Solved Factor | Average Actual Error |
| --- | --- | --- |
| A | 1.95% | 2.15% |
| B | -4.15% | -2.38% |
| C | 8.18% | 5.20% |
| D | -9.47% | -4.97% |
| E | 36.75% | 27.33% |
| F | 0.00% | -0.04% |

The cells highlighted in yellow are the values that were determined by the error minimization solver routine. When comparing the solved error using the GRG Nonlinear method to the average actual error we note:

- The direction of the error (positive vs. negative) always matched the actual error. This was true for this case but is unlikely to be the case for every situation.

- The solver correctly identified the relative contribution of meter error (e.g., that meter E had the largest error).

- The solver slightly underestimated the error contribution of meter A and overestimated the error contribution of meters B through E.

  - This is attributed to the use of a ‘bias only’ solver method when the actual error function used both slope and bias.

Based on initial tests, flow pattern matching methods are likely to be reliable in identifying which meter is contributing to the most to a zone’s lost and unaccounted with a corresponding estimate of the error contribution. This was the original goal for this method. The method would help identify which meters should be field audited with the potential to find actual problems with the measurement that then can be corrected.

The next steps are to evaluate different solver methods for the estimated error (e.g., least square regression) and additional simulated flow cases before evaluating it against subsets of the host site data.

5: Project Schedule

The project is on schedule on a work completed basis despite having some tasks lag behind. Overall, the project is estimated at 35% complete which is ahead of plan of 34% by end of project Q4.